Smart Solar Charge Controller Monitoring Dashboard using ESP32 and React

Technical Project Documentation

1. Abstract

The Smart Solar Charge Controller Monitoring Dashboard is an IoT-based monitoring system developed to simulate and visualize the working of a solar charge controller. The project combines an ESP32-WROOM-32 microcontroller with a React + Vite web dashboard to provide real-time monitoring of important solar and battery parameters over a Wi-Fi network.

The ESP32 hosts a lightweight REST API server that continuously generates simulated operating values such as solar voltage, current, battery status, charging mode, and motor state. The React dashboard fetches these values every second and displays them through an intuitive graphical interface featuring live gauges, status indicators, and control buttons.

The project demonstrates embedded firmware development, wireless communication, REST API implementation, and modern web application development within a single integrated system.

2. Project Overview

The primary objective of this project is to create a real-time monitoring dashboard for a simulated solar charge controller. Instead of relying on physical sensors, the ESP32 internally generates realistic operating values to mimic an actual solar power system.

Users can interact with the dashboard through any device connected to the same Wi-Fi network. The dashboard allows monitoring of:

- Solar Voltage

- Solar Current

- Solar Power

- Battery Voltage

- Battery State of Charge (SOC)

- Battery Temperature

- Charging Status

- Controller Operating Mode

- Motor Status

Additionally, users can simulate various environmental conditions such as Sunny, Cloudy, and Rainy, as well as remotely control the motor using REST API commands.

3. Objectives

The project was developed with the following objectives:

- Design an ESP32-based REST API server for wireless communication.

- Develop a responsive React dashboard for real-time visualization.

- Simulate realistic solar charging conditions.

- Implement motor control through HTTP requests.

- Demonstrate MPPT charging behavior in a simplified simulation.

- Create an easily expandable platform for future hardware integration.

4. System Requirements

Hardware Components

| Component | Description |
| --- | --- |
| ESP32-WROOM-32 | Main controller and REST API server |
| Wi-Fi Network | Communication between ESP32 and dashboard |
| Computer/Laptop | Runs the React dashboard |

Software Components

| Software | Purpose |
| --- | --- |
| Arduino IDE | ESP32 firmware development |
| React.js | Dashboard frontend |
| Vite | React development environment |
| HTML/CSS | User Interface |
| JavaScript | Dashboard logic |
| REST API | Communication protocol |

5. Overall System Architecture

The overall system consists of two primary modules:

Embedded Layer

- ESP32 firmware

- REST API server

- Simulation engine

Application Layer

- React Dashboard

- Live data visualization

- User controls

System Workflow

ESP32 Simulation Engine

│

▼

REST API Server

│

Wi-Fi Network

│

▼

React Dashboard

│

▼

Live Monitoring & Control

6. Working Principle

The ESP32 continuously generates simulated operating values representing a solar charge controller.

Every second, the React dashboard sends a GET /data request to retrieve the latest system parameters.

The received JSON response is processed and displayed as:

- Live Gauges

- Battery Indicators

- Solar Statistics

- Charging Status

- Controller Mode

- Motor Status

Whenever the user selects a weather condition or controls the motor, the dashboard sends another REST API request to the ESP32, which immediately updates the simulation values.

7. ESP32 Firmware Design

The ESP32 firmware performs four primary functions:

REST API Hosting

The ESP32 creates an HTTP server accessible within the local Wi-Fi network.

Simulation Engine

Generates dynamic values including:

- Solar Voltage

- Solar Current

- Battery Voltage

- Battery Temperature

- Charging Status

- Controller Mode

Request Handling

The firmware processes incoming REST API requests and updates the simulation accordingly.

JSON Response Generation

All sensor values are packaged into JSON format and transmitted to the dashboard.

8. REST API Description

| API | Function |
| --- | --- |
| GET /data | Returns all system parameters |
| GET /sunny | Simulates sunny weather |
| GET /cloudy | Simulates cloudy weather |
| GET /rainy | Simulates rainy weather |
| GET /motor/on | Turns motor ON |
| GET /motor/off | Turns motor OFF |

Example JSON Response

{

"solarVoltage": 20.5,

"solarCurrent": 4.2,

"solarPower": 86.1,

"batteryVoltage": 12.8,

"batterySOC": 82,

"batteryTemperature": 31,

"chargingStatus": "Charging",

"controllerMode": "MPPT",

"motorStatus": "OFF"

}

9. Dashboard Architecture

The React dashboard consists of multiple functional sections:

| Module | Function |
| --- | --- |
| Live Gauges | Displays voltage, current, and power |
| Battery Panel | Shows SOC, voltage, and temperature |
| Weather Controls | Changes simulation conditions |
| Motor Controls | ON/OFF control |
| Status Cards | Displays charging state and controller mode |

The dashboard automatically refreshes data every second without requiring user interaction.

10. Simulation Logic

Weather Simulation

Three weather modes are implemented:

| Weather | Solar Output |
| --- | --- |
| Sunny | Maximum solar generation |
| Cloudy | Moderate generation |
| Rainy | Minimum generation |

Changing the weather modifies the simulated voltage and current values, directly affecting charging performance.

Battery Simulation

The battery charge level varies according to the simulated solar power.

- Higher solar power increases SOC.

- Reduced solar power slows charging.

- Temperature changes gradually to mimic realistic battery behavior.

Motor Control

Users can remotely control the motor using the dashboard.

- Motor ON → Simulates power consumption.

- Motor OFF → Stops load consumption.

The ESP32 immediately updates the motor status and reflects the change on the dashboard.

MPPT Charging Logic

The controller operates in MPPT (Maximum Power Point Tracking) mode during charging simulation.

The firmware adjusts charging behavior according to the available solar power, demonstrating efficient energy utilization under varying weather conditions.

11. Testing and Results

Testing Procedure

The following tests were conducted:

| Test | Result |
| --- | --- |
| Wi-Fi Connectivity | Successful |
| REST API Communication | Successful |
| Dashboard Refresh | Updates every second |
| Weather Simulation | Functional |
| Motor Control | Functional |
| MPPT Simulation | Functional |

Observations

- Dashboard maintained stable communication with the ESP32.

- REST API responses were received without noticeable delay.

- Weather changes immediately affected simulated solar values.

- Motor control commands were executed successfully.

- Live gauges refreshed continuously, providing real-time visualization.

12. Challenges and Future Improvements

Challenges Faced

- Maintaining stable Wi-Fi communication during continuous polling.

- Synchronizing dashboard updates with firmware-generated values.

- Designing a responsive dashboard capable of displaying multiple real-time parameters efficiently.

Future Improvements

- Integrate real solar sensors instead of simulated values.

- Add historical data logging using a database.

- Implement user authentication for secure remote access.

- Support cloud connectivity for monitoring over the internet.

- Include fault detection and alert notifications for abnormal operating conditions.

13. Conclusion

The Smart Solar Charge Controller Monitoring Dashboard successfully demonstrates the integration of embedded systems and web technologies for real-time monitoring applications. By combining an ESP32-based REST API server with a React dashboard, the system provides a responsive and user-friendly platform for visualizing solar charging parameters and controlling system functions.

Although the current implementation uses simulated data, the software architecture is modular and can be extended to interface with actual sensors and hardware in future developments. The project effectively showcases practical skills in embedded firmware development, RESTful communication, IoT system design, and modern web application development, making it a strong foundation for industrial monitoring solutions and internship-level engineering projects.